堡盟 Baumer 雷达传感器测量与扁平或圆形物体之间的距离R600V.DAE0-11188367

基本参数

感应范围 Sd                        0,3 … 5 m

重复精度                            < 6,5 mm

温漂                                ≤ ± 3 % Sd (全量程; ± 15 mm)

上电指示灯                          绿色LED

载波频率                            122 … 123 GHz

带宽                                1 GHz

线性误差                            ± 0,3 %

调制类型                            FMCW

发射功率（EIRP）                    < +20 dBm

发散角                              6 °

MTTF                                 > 126 years

认证 / 证书                   FCC / CFR-47 part 18 (USA)

EN / 305 550-1 V.1.2.1 (European Union)

EN / 305 550-2 V.1.2.1 (European Union)

EN 61000-6-2

EN 61000-6-3

EN 50581

EN 13309

EN ISO 14982

ISO 13766

电气参数

电源电压范围 +Vs               9 … 32 VDC

最大电流消耗（无负载）         160 mA

波特率                         250 kBaud (500 kBaud)

output rate                      1 … 100 Hz

短路保护                         是

反极性保护                       是, Vs到 GND

输出电路                         CAN (5 V), CAN SAE J1939

机械参数

类型                            方型 with M6 mounting holes

外壳材质                        PA 10T/X

宽度 / 直径                     97 mm

高度 / 长度                     84 mm

深度                            42,5 mm

连接方式                        连接头 AMPSEAL 16, 4 针

最大紧固扭矩                    15 Nm (A: 12 Nm)

环境条件

工作温度                         -40 … +70 °C

储存温度                         -40 … +85 °C

防护等级                         IP 69K

IP 68

半正弦振动                       50 g / 11 ms resp. 100 g / 6 ms, 18 jolts   per axis and direction

particle impact                  Gravel acc. to EN 11124-2; gravel pressure 2 bar, 2 cycles

random vibration                  IEC 60068-2-64

Frequency : 5 … 2000 Hz

Overall Grms: 11,55 Grms

Axis: all 3 axes / 8h per axis

salt environment                  ISO 9227

48h / +35 °C / pH=6,5 ... 7,2 / 5 % sodium chloride solution

chemical resistance               Brush test with typical chemicals in agriculture, mobile machines (soak 168h)

combined environment               32 VDC / -40 … +85 °C / 0 ... 70 % RH; 100 cycles / 12h per cycle

尺寸图

针角定义